Snow Business ProFlake RTU (WP 1617)

Safety Data Sheet

according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830 Reference number: Periodic review of SDS 09/13/2024

Issue date: 10.01.2023 Version: 1.0

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

Product form	:  Mixture

Product name	:  Snow Business ProFlake RTU (WP 1617)

Product code	:  WP 1617 (RTU)

Type of product	:  Mixture containing mainly :Solvents,Surface-active agents,Chelating agent

Product group	:  Blend

1.2. Relevant identified uses of the substance or mixture and uses advised against

# Relevant identified uses

Industrial/Professional use spec	:  Industrial

For professional use only

Use of the substance/mixture	:  Snow formulation (RTU)

# Uses advised against

No additional information available

1.3. Details of the supplier of the safety data sheet

Wessex Chemical Factors Ltd

17 Crane Way, Woolsbridge Industrial Park,

Three Legged Cross, Wimborne, Dorset – BH21 6FA United Kingdom

T +44 (0) 1202 823 699 - F +44 (0) 1202 813 863

www.wessexchemicalfactors.co.uk

E-mail address of competent person responsible for the SDS : info@wessexchemicalfactors.co.uk

1.4. Emergency telephone number

Emergency number	:  +44 (0) 1202 823 699 (Office hours only 9am - 5pm Monday - Thursday, 9am - 4pm Friday.)

+44 (0) 7973629367 (Out of hours emergency number)

SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

# Classification according to Regulation (EC) No. 1272/2008 [CLP]

Not classified

# Adverse physicochemical, human health and environmental effects

No additional information available

2.2. Label elements

# Labelling according to Regulation (EC) No. 1272/2008 [CLP]

EUH-statements	:  EUH210 - Safety data sheet available on request.

2.3. Other hazards

No additional information available

SECTION 3: Composition/information on ingredients

3.1. Substances

Not applicable

3.2. Mixtures

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| propan-2-ol; isopropyl alcohol; isopropanol | CAS-No.: 67-63-0 EC-No.: 200-661-7 EC Index-No.: 603-117-00-0 REACH-no: 01-2119457558- 25-XXXX | 1 – 3 | Flam. Liq. 2, H225 Eye Irrit. 2, H319 STOT SE 3, H336 |
| sodium dodecylbenzenesulphonate | CAS-No.: 25155-30-0 EC-No.: 246-680-4 | 0.1 – 1 | Acute Tox. 4 (Oral), H302 Skin Irrit. 2, H315 Eye Dam. 1, H318 |
| 1-propanaminium, 3-amino-N-(carboxymethyl)-N,N- dimethyl-, N-C8-18(even numbered) acyl derivs., hydroxides, inner salts < 4 % | CAS-No.: 97862-59-4 EC-No.: 931-296-8 | 0.1 – 1 | Aquatic Chronic 3, H412 |
| reaction mass of (2S)-alanine, N,N- bis(carboxymethyl)-, trisodium salt and (2R)-alanine, N,N-bis(carboxymethyl)-, trisodium salt | EC-No.: 423-270-5 REACH-no: 01-0000016977- 53 | 0.1 – 1 | Met. Corr. 1, H290 |
| D-glucopyranose, oligomeric, C10-16(even numbered) alkyl glycosides | CAS-No.: 110615-47-9 EC-No.: 600-975-8 | 0.01 – 1 | Skin Irrit. 2, H315 Eye Dam. 1, H318 |
| propyl 4-hydroxybenzoate | CAS-No.: 94-13-3 EC-No.: 202-307-7 | 0.01 – 0.1 | Not classified |
| butyl 4-hydroxybenzoate substance listed as REACH Candidate (butyl 4- hydroxybenzoate) | CAS-No.: 94-26-8 EC-No.: 202-318-7 | 0.01 – 0.1 | Not classified |
| methyl 4-hydroxybenzoate | CAS-No.: 99-76-3 EC-No.: 202-785-7 | < 0.01 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 STOT SE 3, H335 Aquatic Chronic 3, H412 |
| antifoam emulsion B | - | < 0.01 | Not classified |

Full text of H- and EUH-statements: see section 16

SECTION 4: First aid measures

4.1. Description of first aid measures

First-aid measures general	:  Never give anything by mouth to an unconscious person. If you feel unwell, seek medical

advice (show the label where possible).

First-aid measures after inhalation	:  Allow affected person to breathe fresh air. Allow the victim to rest.

First-aid measures after skin contact	:  Remove affected clothing and wash all exposed skin area with mild soap and water, followed by warm water rinse.

First-aid measures after eye contact	:  Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or redness

persists.

First-aid measures after ingestion	:  Rinse mouth. Do NOT induce vomiting. Obtain emergency medical attention.

4.2. Most important symptoms and effects, both acute and delayed

Symptoms/effects	:  Not expected to present a significant hazard under anticipated conditions of normal use.

4.3. Indication of any immediate medical attention and special treatment needed

No additional information available

SECTION 5: Firefighting measures

5.1. Extinguishing media

Suitable extinguishing media	:  Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media	:  Do not use a heavy water stream.

5.2. Special hazards arising from the substance or mixture

No additional information available

5.3. Advice for firefighters

Firefighting instructions	:  Use water spray or fog for cooling exposed containers. Exercise caution when fighting any chemical fire. Prevent fire fighting water from entering the environment.

Protection during firefighting	:  Do not enter fire area without proper protective equipment, including respiratory protection.

SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

# For non-emergency personnel

Emergency procedures	:  Evacuate unnecessary personnel.

# For emergency responders

Protective equipment	:  Equip cleanup crew with proper protection.

Emergency procedures	:  Ventilate area.

6.2. Environmental precautions

Prevent entry to sewers and public waters. Notify authorities if liquid enters sewers or public waters.

6.3. Methods and material for containment and cleaning up

Methods for cleaning up	:  Soak up spills with inert solids, such as clay or diatomaceous earth as soon as possible.

Collect spillage. Store away from other materials.

6.4. Reference to other sections

See Heading 8. Exposure controls and personal protection.

SECTION 7: Handling and storage

7.1. Precautions for safe handling

Precautions for safe handling	:  Wash hands and other exposed areas with mild soap and water before eating, drinking or

smoking and when leaving work. Provide good ventilation in process area to prevent formation of vapour.

7.2. Conditions for safe storage, including any incompatibilities

Storage conditions	:  Keep only in the original container in a cool, well ventilated place away from : Keep container closed when not in use.

Incompatible products	:  Strong bases. Strong acids.

Incompatible materials	:  Sources of ignition. Direct sunlight.

7.3. Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

# 8.1.1 National occupational exposure and biological limit values

| propan-2-ol; isopropyl alcohol; isopropanol (67-63-0) |  |
| --- | --- |
| United Kingdom - Occupational Exposure Limits |  |
| Local name | Propan-2-ol |
| WEL TWA (OEL TWA) [1] | 999 mg/m³ |
| WEL TWA (OEL TWA) [2] | 400 ppm |
| WEL STEL (OEL STEL) | 1250 mg/m³ |
| WEL STEL (OEL STEL) [ppm] | 500 ppm |
| Regulatory reference | EH40/2005 (Fourth edition, 2020). HSE |

    - Recommended monitoring procedures

No additional information available

# Air contaminants formed

No additional information available

# DNEL and PNEC

No additional information available

# Control banding

No additional information available

8.2. Exposure controls

# Appropriate engineering controls

No additional information available

# Personal protection equipment Personal protective equipment:

Avoid all unnecessary exposure.

# Eye and face protection Eye protection:

Even though no specific eye irritation data are available, wear eye protection appropriate to conditions of use when handling this material

# Skin protection Hand protection:

Wear protective gloves.

# Respiratory protection Respiratory protection:

No special protection required where adequate ventilation is maintained

# Thermal hazards

No additional information available

# 8.2.3. Environmental exposure controls Other information:

Do not eat, drink or smoke during use.

SECTION 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

Physical state	:  Liquid

Colour	:  Colourless.

Odour	:  characteristic.

Odour threshold	:  No data available

pH	:  No data available

Relative evaporation rate (butylacetate=1)	:  No data available Melting point	:  No data available

Freezing point	:  No data available

Boiling point	:  No data available

Flash point	:  65 °C

Auto-ignition temperature	:  No data available

Decomposition temperature	:  No data available

Flammability (solid, gas)	:  Non flammable.

Vapour pressure	:  No data available

Relative vapour density at 20 °C	:  No data available

Relative density	:  No data available

Density	:  ~ 1 g/cm³

Solubility	:  No data available

Partition coefficient n-octanol/water (Log Pow)	:  No data available Viscosity, kinematic	:  No data available

Viscosity, dynamic	:  No data available

Explosive properties	:  No data available

Oxidising properties	:  No data available

Explosive limits	:  No data available

9.2. Other information

No additional information available

SECTION 10: Stability and reactivity

10.1. Reactivity

No additional information available

10.2. Chemical stability

Stable under normal conditions of use.

10.3. Possibility of hazardous reactions

Not established.

10.4. Conditions to avoid

Direct sunlight. Extremely high or low temperatures.

10.5. Incompatible materials

Strong acids. Strong bases.

10.6. Hazardous decomposition products

fume. Carbon monoxide. Carbon dioxide.

SECTION 11: Toxicological information

11.1 Information on toxicological effects

Acute toxicity (oral)	:  Not classified

Acute toxicity (dermal)	:  Not classified

| Acute toxicity (inhalation) : Not classified |  |
| --- | --- |
| sodium dodecylbenzenesulphonate (25155-30-0) |  |
| LD50 oral rat | 500 – 2000 mg/kg bodyweight |
| propyl 4-hydroxybenzoate (94-13-3) |  |
| LD50 oral rat | > 5000 mg/kg |
| LD50 oral | > 8000 mg/kg (Mouse) |
| propan-2-ol; isopropyl alcohol; isopropanol (67-63-0) |  |
| LD50 oral rat | 5045 mg/kg |
| LD50 dermal rabbit | 12800 mg/kg |
| LC50, male, female, Inhalation, rat | > 10000 ppm (6 Hours, (OECD 403 method)) |
| methyl 4-hydroxybenzoate (99-76-3) |  |
| LD50 oral rat | 2100 mg/kg |
| reaction mass of (2S)-alanine, N,N-bis(carboxymethyl)-, trisodium salt and (2R)-alanine, N,N-bis(carboxymethyl)-, trisodium salt |  |
| LD50 oral rat | > 4000 mg/kg |
| LD50 dermal rat | > 4000 mg/kg |
| LC50 Inhalation - Rat (Dust/Mist) | > 5 mg/l/4h |
| D-glucopyranose, oligomeric, C10-16(even numbered) alkyl glycosides (110615-47-9) |  |
| LD50 oral rat | > 5000 mg/kg bodyweight |
| LD50 dermal rabbit | > 2000 mg/kg bodyweight |
| 1-propanaminium, 3-amino-N-(carboxymethyl)-N,N-dimethyl-, N-C8-18(even numbered) acyl derivs., hydroxides, inner salts < 4 % (97862-59-4) |  |
| LD50 oral rat | > 4900 mg/kg |

Skin corrosion/irritation	:  Not classified

Additional information	:  Based on available data, the classification criteria are not met

Serious eye damage/irritation	:  Not classified

Additional information	:  Based on available data, the classification criteria are not met

Respiratory or skin sensitisation	:  Not classified

Additional information	:  Based on available data, the classification criteria are not met

Germ cell mutagenicity	:  Not classified

Additional information	:  Based on available data, the classification criteria are not met

Carcinogenicity	:  Not classified

Additional information	:  Based on available data, the classification criteria are not met

Reproductive toxicity	:  Not classified

Additional information	:  Based on available data, the classification criteria are not met

STOT-single exposure	:  Not classified

Additional information	:  Based on available data, the classification criteria are not met

| propan-2-ol; isopropyl alcohol; isopropanol (67-63-0) |  |
| --- | --- |
| STOT-single exposure | May cause drowsiness or dizziness. |
| methyl 4-hydroxybenzoate (99-76-3) |  |
| STOT-single exposure | May cause respiratory irritation. |

STOT-repeated exposure	:  Not classified

Additional information	:  Based on available data, the classification criteria are not met

Aspiration hazard	:  Not classified

Additional information	:  Based on available data, the classification criteria are not met

Potential adverse human health effects and symptoms

:  Based on available data, the classification criteria are not met

SECTION 12: Ecological information

12.1. Toxicity

Hazardous to the aquatic environment, short-term (acute)

Hazardous to the aquatic environment, long-term (chronic)

:  Not classified

:  Not classified

| sodium dodecylbenzenesulphonate (25155-30-0) |  |
| --- | --- |
| LC50 - Fish [1] | 3.2 – 5.6 mg/l (Rainbow trout) |
| EC50 72h - Algae [1] | 17.03 mg/l |
| propyl 4-hydroxybenzoate (94-13-3) |  |
| LC50 - Fish [1] | 6.4 mg/l (Zebrafish) |
| EC50 - Crustacea [1] | 15.4 mg/l |
| EC50 72h - Algae [1] | 7.6 mg/l |
| propan-2-ol; isopropyl alcohol; isopropanol (67-63-0) |  |
| LC50 - Fish [1] | 9640 mg/l Fathead minnow (Pimephales promelas) |
| EC50 72h - Algae [1] | > 1000 mg/l (Desmodesmus subspicatus) |
| EC50, daphnia, short term | 9714 mg/l (24 Hours, (OECD 202 method)) |
| EC5, microorganisms, Pseudomonas putida | 1050 mg/l (16 Hours) |
| methyl 4-hydroxybenzoate (99-76-3) |  |
| LC50 - Fish [1] | 59.5 mg/l (Japanese rice fish) |
| EC50 - Crustacea [1] | 11.2 mg/l |
| EC50 72h - Algae [1] | 91 mg/l |
| reaction mass of (2S)-alanine, N,N-bis(carboxymethyl)-, trisodium salt and (2R)-alanine, N,N-bis(carboxymethyl)-, trisodium salt |  |
| LC50 - Fish [1] | > 200 mg/l (Zebrafish) |
| EC50 - Crustacea [1] | > 200 mg/l |
| EC50 72h - Algae [1] | > 200 mg/l |
| NOEC chronic fish | ≥ 200 mg/l (28 d, Rainbow trout) |
| NOEC chronic crustacea | ≥ 200 mg/l (21 d) |
| D-glucopyranose, oligomeric, C10-16(even numbered) alkyl glycosides (110615-47-9) |  |
| LC50 - Fish [1] | 2.95 mg/l (Zebrafish) |
| EC50 - Crustacea [1] | 7 mg/l |
| 1-propanaminium, 3-amino-N-(carboxymethyl)-N,N-dimethyl-, N-C8-18(even numbered) acyl derivs., hydroxides, inner salts < 4 % (97862-59-4) |  |
| LC50 - Fish [1] | 1.11 mg/l |
| EC50 - Crustacea [1] | 7 |
| EC50 72h - Algae [1] | 2.4 mg/l |
| NOEC chronic fish | 0.16 mg/l |

12.2. Persistence and degradability

| Snow Business ProFlake RTU (WP 1617) |  |
| --- | --- |
| Persistence and degradability | Not established. |
| sodium dodecylbenzenesulphonate (25155-30-0) |  |
| Persistence and degradability | Readily biodegradable. |
| propyl 4-hydroxybenzoate (94-13-3) |  |
| Persistence and degradability | Readily biodegradable. |
| propan-2-ol; isopropyl alcohol; isopropanol (67-63-0) |  |
| Persistence and degradability | Readily biodegradable. |
| methyl 4-hydroxybenzoate (99-76-3) |  |
| Persistence and degradability | Readily biodegradable. |
| Biodegradation | 89 – 92.2 % (28 days) |
| reaction mass of (2S)-alanine, N,N-bis(carboxymethyl)-, trisodium salt and (2R)-alanine, N,N-bis(carboxymethyl)-, trisodium salt |  |
| Persistence and degradability | Readily biodegradable. |
| BOD (% of ThOD) | 80 – 90 % ThOD |
| D-glucopyranose, oligomeric, C10-16(even numbered) alkyl glycosides (110615-47-9) |  |
| Persistence and degradability | Readily biodegradable. |
| 1-propanaminium, 3-amino-N-(carboxymethyl)-N,N-dimethyl-, N-C8-18(even numbered) acyl derivs., hydroxides, inner salts < 4 % (97862-59-4) |  |
| Persistence and degradability | Readily biodegradable. |
| Biodegradation | > 60 % (28 days) |

12.3. Bioaccumulative potential

| Snow Business ProFlake RTU (WP 1617) |  |
| --- | --- |
| Bioaccumulative potential | Not established. |
| propyl 4-hydroxybenzoate (94-13-3) |  |
| Partition coefficient n-octanol/water (Log Pow) | 3.04 |
| Partition coefficient n-octanol/water (Log Kow) | 2.8 |
| propan-2-ol; isopropyl alcohol; isopropanol (67-63-0) |  |
| Partition coefficient n-octanol/water (Log Pow) | 0.05 |
| Bioaccumulative potential | No bioaccumulation. |
| reaction mass of (2S)-alanine, N,N-bis(carboxymethyl)-, trisodium salt and (2R)-alanine, N,N-bis(carboxymethyl)-, trisodium salt |  |
| Partition coefficient n-octanol/water (Log Pow) | -4 |
| Bioaccumulative potential | Based on the n-octanol/water partition coefficient accumulation in organisms is not expected. |

12.4. Mobility in soil

| propan-2-ol; isopropyl alcohol; isopropanol (67-63-0) |  |
| --- | --- |
| Surface tension | 22.7 mN/m |
| Ecology - soil | Very mobile. Soluble material/quickly disperses in water. |
| reaction mass of (2S)-alanine, N,N-bis(carboxymethyl)-, trisodium salt and (2R)-alanine, N,N-bis(carboxymethyl)-, trisodium salt |  |
| Surface tension | 71.5 mN/m @ 1g/L |

12.5. Results of PBT and vPvB assessment

| Component |  |
| --- | --- |
| propan-2-ol; isopropyl alcohol; isopropanol (67-63-0) | This substance/mixture does not meet the PBT criteria of REACH regulation, annex XIII This substance/mixture does not meet the vPvB criteria of REACH regulation, annex XIII |
| 1-propanaminium, 3-amino-N-(carboxymethyl)-N,N- dimethyl-, N-C8-18(even numbered) acyl derivs., hydroxides, inner salts < 4 % (97862-59-4) | This substance/mixture does not meet the PBT criteria of REACH regulation, annex XIII This substance/mixture does not meet the vPvB criteria of REACH regulation, annex XIII |
| reaction mass of (2S)-alanine, N,N- bis(carboxymethyl)-, trisodium salt and (2R)-alanine, N,N-bis(carboxymethyl)-, trisodium salt | This substance/mixture does not meet the PBT criteria of REACH regulation, annex XIII This substance/mixture does not meet the vPvB criteria of REACH regulation, annex XIII |
| D-glucopyranose, oligomeric, C10-16(even numbered) alkyl glycosides (110615-47-9) | This substance/mixture does not meet the PBT criteria of REACH regulation, annex XIII This substance/mixture does not meet the vPvB criteria of REACH regulation, annex XIII |
| propyl 4-hydroxybenzoate (94-13-3) | This substance/mixture does not meet the PBT criteria of REACH regulation, annex XIII This substance/mixture does not meet the vPvB criteria of REACH regulation, annex XIII |
| butyl 4-hydroxybenzoate (94-26-8) | This substance/mixture does not meet the PBT criteria of REACH regulation, annex XIII This substance/mixture does not meet the vPvB criteria of REACH regulation, annex XIII |
| methyl 4-hydroxybenzoate (99-76-3) | This substance/mixture does not meet the PBT criteria of REACH regulation, annex XIII This substance/mixture does not meet the vPvB criteria of REACH regulation, annex XIII |

12.6. Other adverse effects

Additional information	:  Avoid release to the environment.

SECTION 13: Disposal considerations

13.1. Waste treatment methods

Product/Packaging disposal recommendations	:  Dispose in a safe manner in accordance with local/national regulations. Ecology - waste materials	:  Avoid release to the environment.

SECTION 14: Transport information

In accordance with ADR / IMDG / IATA / ADN / RID

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.1. UN number |  |  |  |  |
| Not regulated | Not regulated | Not regulated | Not regulated | Not regulated |
| 14.2. UN proper shipping name |  |  |  |  |
| Not regulated | Not regulated | Not regulated | Not regulated | Not regulated |
| 14.3. Transport hazard class(es) |  |  |  |  |
| Not regulated | Not regulated | Not regulated | Not regulated | Not regulated |

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.4. Packing group |  |  |  |  |
| Not regulated | Not regulated | Not regulated | Not regulated | Not regulated |
| 14.5. Environmental hazards |  |  |  |  |
| Not regulated | Not regulated | Not regulated | Not regulated | Not regulated |
| No supplementary information available |  |  |  |  |

14.6. Special precautions for user

# Overland transport

Not regulated

# Transport by sea

Not regulated

# Air transport

Not regulated

# Inland waterway transport

Not regulated

# Rail transport

Not regulated

14.7. Transport in bulk according to Annex II of Marpol and the IBC Code

Not applicable

SECTION 15: Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

# EU-Regulations

Contains no REACH substances with Annex XVII restrictions Contains no substance on the REACH candidate list ≥ 0,1 % / SCL Contains no REACH Annex XIV substances

Contains no substance subject to Regulation (EU) No 649/2012 of the European Parliament and of the Council of 4 July 2012 concerning the export and import of hazardous chemicals.

Contains no substance subject to Regulation (EU) No 2019/1021 of the European Parliament and of the Council of 20 June 2019 on persistent organic pollutants

Contains no substance subject to Regulation (EU) 2019/1148 of the European Parliament and of the Council of 20 June 2019 on the marketing and use of explosives precursors.

# National regulations

No additional information available

15.2. Chemical safety assessment

No chemical safety assessment has been carried out

SECTION 16: Other information

Data sources	:  REGULATION (EC) No 1272/2008 OF THE EUROPEAN PARLIAMENT AND OF THE

COUNCIL of 16 December 2008 on classification, labelling and packaging of substances and mixtures, amending and repealing Directives 67/548/EEC and 1999/45/EC, and amending Regulation (EC) No 1907/2006.

Other information	:  None.

| Full text of H- and EUH-statements: |  |
| --- | --- |
| Acute Tox. 4 (Oral) | Acute toxicity (oral), Category 4 |
| Aquatic Chronic 3 | Hazardous to the aquatic environment — Chronic Hazard, Category 3 |
| EUH210 | Safety data sheet available on request. |
| Eye Dam. 1 | Serious eye damage/eye irritation, Category 1 |
| Eye Irrit. 2 | Serious eye damage/eye irritation, Category 2 |
| Flam. Liq. 2 | Flammable liquids, Category 2 |
| H225 | Highly flammable liquid and vapour. |
| H290 | May be corrosive to metals. |
| H302 | Harmful if swallowed. |
| H315 | Causes skin irritation. |
| H318 | Causes serious eye damage. |
| H319 | Causes serious eye irritation. |
| H335 | May cause respiratory irritation. |
| H336 | May cause drowsiness or dizziness. |
| H412 | Harmful to aquatic life with long lasting effects. |
| Met. Corr. 1 | Corrosive to metals, Category 1 |
| Skin Irrit. 2 | Skin corrosion/irritation, Category 2 |
| STOT SE 3 | Specific target organ toxicity — Single exposure, Category 3, Narcosis |

Safety Data Sheet (SDS), EU

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.